SECTION 01731 - CUTTING AND PATCHING

GENERAL

RELATED DOCUMENTS

Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 1 Sections, apply to this Section.

SUMMARY

This Section includes procedural requirements for cutting and patching.

See Divisions 2 through 16 Sections for specific requirements and limitations applicable to cutting and patching individual parts of the Work.

Requirements in this Section apply to mechanical and electrical installations.  See Divisions 15 and 16 Sections for other requirements and limitations applicable to cutting and patching mechanical and electrical installations.

The contractor shall be responsible for all cutting, fitting or patching that may be required to complete the work or to make its several parts fit together properly.

The contractor shall not damage or endanger any portion of the work or the work of the Owner or any separate contractors by cutting, patching or otherwise altering any work, or by excavation.  The contractor shall not cut or otherwise alter the work of the Owner or any separate contractor except with the written consent of the Owner and of such separate contractor.  The contractor shall not unreasonably withhold from the Owner or any separate contractor his consent to cutting or otherwise altering the work.

SUBMITTALS

Cutting and Patching Proposal:  Submit a proposal describing procedures at least 10 days before the time cutting and patching will be performed, requesting approval to proceed.  Include the following information:

Extent:  Describe cutting and patching, show how they will be performed, and indicate why they cannot be avoided.

Changes to Existing Construction:  Describe anticipated results.  Include changes to structural elements and operating components as well as changes in building's appearance and other significant visual elements.

Products:  List products to be used and firms or entities that will perform the Work.

Dates:  Indicate when cutting and patching will be performed.

Utilities:  List utilities that cutting and patching procedures will disturb or affect.  List utilities that will be relocated and those that will be temporarily out of service.  Indicate how long service will be disrupted.

Structural Elements:  Where cutting and patching involve adding reinforcement to structural elements, submit details and engineering calculations showing integration of reinforcement with original structure.

Owner Approval:  Obtain approval of cutting and patching proposal before cutting and patching.  Approval does not waive right to later require removal and replacement of unsatisfactory work.

QUALITY ASSURANCE

Structural Elements:  Do not cut and patch structural elements in a manner that could change their load-carrying capacity or load-deflection ratio.

Operational Elements:  Do not cut and patch operating elements and related components in a manner that results in reducing their capacity to perform as intended or that results in increased maintenance or decreased operational life or safety.

Miscellaneous Elements:  Do not cut and patch the following elements or related components in a manner that could change their load-carrying capacity, that results in reducing their capacity to perform as intended, or that results in increased maintenance or decreased operational life or safety.

Visual Requirements:  Do not cut and patch construction in a manner that results in visual evidence of cutting and patching.  Do not cut and patch construction exposed on the exterior or in occupied spaces in a manner that would, in Owner or Architect's opinion, reduce the building's aesthetic qualities.  Remove and replace construction that has been cut and patched in a visually unsatisfactory manner.

If possible, retain original Installer or fabricator to cut and patch exposed Work listed below.  If it is impossible to engage original Installer or fabricator, engage another recognized, experienced, and specialized firm.

WARRANTY

Existing Warranties:  Remove, replace, patch, and repair materials and surfaces cut or damaged during cutting and patching operations, by methods and with materials so as not to void existing warranties.

PRODUCTS

MATERIALS

General:  Comply with requirements specified in other Sections of these Specifications.

Existing Materials:  Use materials identical to existing materials.  For exposed surfaces, use materials that visually match existing adjacent surfaces to the fullest extent possible.

If identical materials are unavailable or cannot be used, use materials that, when installed, will match the visual and functional performance of existing materials.

EXECUTION

EXAMINATION

Examine surfaces to be cut and patched and conditions under which cutting and patching are to be performed.

Compatibility:  Before patching, verify compatibility with and suitability of substrates, including compatibility with existing finishes or primers.

Proceed with installation only after unsafe or unsatisfactory conditions have been corrected.

PREPARATION

Temporary Support:  Provide temporary support of Work to be cut.

Protection:  Protect existing construction during cutting and patching to prevent damage.  Provide protection from adverse weather conditions for portions of Project that might be exposed during cutting and patching operations.

PERFORMANCE

General:  Employ skilled workers to perform cutting and patching.  Proceed with cutting and patching at the earliest feasible time, and complete without delay.

Cut existing construction to provide for installation of other components or performance of other construction, and subsequently patch as required to restore surfaces to their original condition.

Cutting:  Cut existing construction by sawing, drilling, breaking, chipping, grinding, and similar operations, including excavation, using methods least likely to damage elements retained or adjoining construction.  If possible, review proposed procedures with original Installer; comply with original Installer's written recommendations.

In general, use hand or small power tools designed for sawing and grinding, not hammering and chopping.  Cut holes and slots as small as possible, neatly to size required, and with minimum disturbance of adjacent surfaces.  Temporarily cover openings when not in use.

Existing Finished Surfaces:  Cut or drill from the exposed or finished side into concealed surfaces.

Concrete and Masonry:  Cut using a cutting machine, such as an abrasive saw or a diamond-core drill.

Excavating and Backfilling:  Comply with requirements in applicable Division 2 Sections where required by cutting and patching operations.

Mechanical and Electrical Services:  Cut off pipe or conduit in walls or partitions to be removed.  Cap, valve, or plug and seal remaining portion of pipe or conduit to prevent entrance of moisture or other foreign matter after cutting.

Proceed with patching after construction operations requiring cutting are complete.

Patching:  Patch construction by filling, repairing, refinishing, closing up, and similar operations following performance of other Work.  Patch with durable seams that are as invisible as possible.  Provide materials and comply with installation requirements specified in other Sections of these Specifications.

Inspection:  Where feasible, test and inspect patched areas after completion to demonstrate integrity of installation.

Exposed Finishes:  Restore exposed finishes of patched areas and extend finish restoration into retained adjoining construction in a manner that will eliminate evidence of patching and refinishing.

Floors and Walls:  Where walls or partitions that are removed extend one finished area into another, patch and repair floor and wall surfaces in the new space.  Provide an even surface of uniform finish, color, texture, and appearance.  Remove existing floor and wall coverings and replace with new materials, if necessary, to achieve uniform color and appearance.

Ceilings:  Patch, repair, or rehang existing ceilings as necessary to provide an even-plane surface of uniform appearance.

Exterior Building Enclosure:  Patch components in a manner that restores enclosure to a weathertight condition.

END OF SECTION 01731

This Page Left Intentionally Blank