SECTION 02000 – Project special provisions

GENERAL

      - SUMMARY

The following project special provisions and/or revisions to Lowe’s Division 2 Standard Site Specifications shall be followed in strict accordance, with the below referenced requirements, during proposed construction.  Project Special Provisions shall supersede standard specification requirements following Lowe’s approval and shall be provided/printed and forwarded on yellow correspondence.  Conformance submittals shall be revised as required to meet governing authority requirements/specifications.

REVISIONS AND ADDITIONS TO STANDARD SPECIFICATIONS

      - Storm Water Pollution Prevention Plan

A site specific Storm Water Pollution Prevention Plan for Lowe’s of [insert name and state of project] has been prepared in accordance with Lowe’s Development Criteria Section 800 and [insert name of governing NPDES Stormwater General Permit for Construction Activities] and is included after Section 02000 – Project Special Provisions and shall be considered part of the contract documents..

(All italicized items shown below this point are for format and reference only and shall be removed and deleted prior to issuance of Project Special Provisions.)

SECTION 02305 - TRENCH EXCAVATION AND BACKFILL, Article 3.02, Item J

Shall be revised to read: (Use bold print to identify revisions to Standard Specification)

Water Mains:  60 inches to top of pipe barrel or 6 inches below the frost line (established by the regulatory authorities having jurisdiction ), whichever is deeper, or as specified on the plans;

- (For example only – 60 inches provided in lieu of Lowe’s Standard Specification of 42 inches for water main coverage)

SECTION 02741- HOT-MIX ASPHALT PAVING, Article 3.05, Item E

Add: (Use bold print to identify information added to Standard Specification)

- Density:  Compare density of in-place material against laboratory specimen of same mixture, subjected to 50 blows of a Standard Marshall hammer on each side of specimen.  Densities for in-place surface course materials shall be compared to the laboratory specimen of same mixture, subjected to 75 blows of a Standard Marshall hammer on each side of the specimen.  Minimum acceptable density of in-place material shall be 95% of recorded laboratory specimen density.

- SECTION 02000A	SOIL CEMENT STABILIZATION

- PROVIDE SPECIFICATION

- SECTION 02000B	UNDERGROUND STORMWATER DETENTION

- PROVIDE SPECIFICATION

END OF SECTION 02000