- SECTION 02361 - TERMITE CONTROL

- GENERAL

RELATED DOCUMENTS

Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 2 Specification Sections, apply to this Section.

SUMMARY

This Section includes the following for termite control:

Soil treatment.

DEFINITIONS

EPA:  Environmental Protection Agency.

PCO:  Pest control operator.

SUBMITTALS

The General Contractor and the Sub-contractor shall execute the provided Conformance Submittal for the product(s) specified.

The General Contractor and Sub-Contractor shall utilize ONLY the products as noted in this specification.  NO SUBSTITUTIONS ARE ALLOWED. The General Contractor shall complete, sign and submit conformance submittal attached to this section.

Soil Treatment Application Report:  After application of treatment is completed, submit report for Owner's permanent record, including the following (as applicable):

Date and time of application.

Moisture content of soil before application.

Brand name and manufacturer of termiticide.

Quantity of undiluted termiticide used.

Dilutions, methods, volumes, and rates of application used.

Areas of application.

Water source for application.

Warranties:  Warranties specified in this Section.

QUALITY ASSURANCE

Applicator Qualifications:  A PCO who is licensed according to regulations of authorities having jurisdiction to apply termite control treatment in jurisdiction where Project is located and who is an experienced installer who has completed termite control treatment similar to that indicated for this Project and employs workers trained and approved by bait station system manufacturer to install manufacturer's products and whose work has a record of successful in-service performance.

Regulatory Requirements:  Regulation of formulation and application of termiticides, labeled with a Federal registration number, in compliance with EPA, State and local regulations governed by authorities having jurisdiction.

PROJECT CONDITIONS

Environmental Limitations:  To ensure penetration, do not treat soil that is water saturated or frozen.  Do not treat soil while precipitation is occurring.  Comply with EPA-Registered Label requirements and requirements of authorities having jurisdiction.

COORDINATION

Coordinate soil treatment application with excavating, filling, and grading and concreting operations.  Treat soil under footings, grade beams, and ground-supported slabs, before construction.

WARRANTY

General Warranty:  The warranty specified in this Article shall not deprive Owner of other rights Owner may have under other provisions of the Contract Documents and shall be in addition to, and run concurrent with, other warranties made by Contractor under requirements of the Contract Documents.

Warranty:  Written warranty for five years from date of Substantial Completion, signed by applicator and Contractor certifying that termite control work, consisting of applied soil termiticide treatment, will prevent infestation of subterranean termites.  If subterranean termite activity or damage is discovered during warranty period, re-treat soil and repair or replace damage caused by termite infestation.

MAINTENANCE SERVICE

Continuing Service:  Provide a proposal for continuing service, including monitoring, inspection, and retreatment for occurrences of termite activity, from applicator to Owner, in the form of a standard yearly (or other period) continuing service agreement, starting on the date of Substantial Completion.  State services, obligations, conditions, and terms for agreement period and for future renewal options.

- PRODUCTS

SOIL TREATMENT

Termiticide:  Provide an EPA-registered termiticide complying with requirements of authorities having jurisdiction, in a soluble or emulsible, concentrated formulation that dilutes with water or foaming agent, and formulated to prevent termite infestation.  Use only soil treatment solutions that are not harmful to plants.  Provide quantity required for application at the label volume and rate for the maximum termiticide concentration allowed for each specific use, according to the product's EPA-Registered Label.

Available Manufacturers:  Subject to compliance with requirements, manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following:

Manufacturers:  Subject to compliance with requirements, provide products by one of the following:

BASF Corporation.

Bayer Corporation.

Dow AgroSciences.

FMC Corporation.

Syngenta.

- EXECUTION

EXAMINATION

Examine substrates, areas, and conditions, with Applicator present, for compliance with requirements for moisture content of the soil, interfaces with earthwork, slab and foundation work, landscape planting, and other conditions affecting performance of termite control.  Proceed with application only after unsatisfactory conditions have been corrected.

PREPARATION

General:  Comply manufacturer's written instructions, EPA, State and local authorities having jurisdiction whichever requirements are most stringent regarding preparation of substrate and removal of of and with e all extraneous sources of wood cellulose and other edible materials such as wood debris, tree stumps and roots, stakes, formwork, and construction waste wood from soil and around foundations.

Soil Treatment Preparation:  Remove foreign matter and impermeable soil materials that could decrease treatment effectiveness on areas to be treated.  Loosen, rake, and level soil to be treated, except previously compacted areas under slabs and footings.  Termiticides may be applied before placing compacted fill under slabs if recommended by termiticide manufacturer.

Fit filling hose connected to water source at the site with a backflow preventer, complying with requirements of authorities having jurisdiction.

APPLICATION, GENERAL

General:  Comply with the most stringent requirements of authorities having jurisdiction and with manufacturer's EPA-Registered Label for products.

APPLYING SOIL TREATMENT

Application:  Mix soil treatment termiticide solution to a uniform consistency.  Provide quantity required for application at the label volume and rate for the maximum specified concentration of termiticide, according to manufacturer's EPA-Registered Label, to the following so that a continuous horizontal and vertical termiticidal barrier or treated zone is established around and under building construction.  Distribute the treatment evenly.

Slabs-on-Grade and Basement Slabs:  Consider underground-supported slab construction, including footings, building slabs, and attached slabs as an overall treatment.  Treat soil materials before concrete footings and slabs are placed.

Foundations:  Treat adjacent soil including soil along entire inside perimeter of foundation walls, along both sides of interior partition walls, around plumbing pipes and electric conduit penetrating slab, and around interior column footers, piers, and along entire outside perimeter, from grade to bottom of footing.  Avoid soil washout around footings.

Penetrations:  At expansion joints, control joints, and areas where slabs will be penetrated.

Avoid disturbance of treated soil after application.  Keep off treated areas until completely dry.

Protect termiticide solution from being diluted until ground-supported slabs are installed. Prevent dispersal of termiticide in treated soils and fills,.  Use waterproof barrier according to EPA-Registered Label instructions.

Post warning signs in areas of application.

Reapply soil treatment solution to areas disturbed by subsequent excavation, grading, landscape planting, or other construction activities following application.

- END OF SECTION 02361

This Page Left Intentionally Blank

# CONFORMANCE SUBMITTAL

# Section 02361 – Termite Control

Lowe’s of ____________________________________________________________

(City, State)

General Contractor:	____________________________________________________

(Company Name, Phone Number)

____________________________________________________

(Address)

Sub-Contractor:		____________________________________________________

(Company Name, Phone Number)

____________________________________________________

(Address)

The following product has been selected (check one box) for use in this project from the list of acceptable products specified:

BASF Corporation.

Bayer Corporation.

Dow AgroSciences.

FMC Corporation.

Syngenta.

I represent to Lowe’s that the product selected will be or has been installed in compliance with the applicable codes for the authorities having jurisdiction and in accordance with the project specification.  If noncompliance is discovered the General Contractor shall make or cause to be made all corrective actions necessary to meet the applicable codes and specifications.  All necessary corrective action shall be completed immediately or as directed or scheduled by Lowe’s without additional cost to the Lowe’s and / or the contract.

- General Contractor:

- _____________________________________________________________

(Signature of the Authorized Agent of the General Contractor)

_____________________________________________________________

(Print Name of the Authorized Agent of the General Contractor)

I represent to Lowe’s that the product selected will be installed in compliance with the applicable codes for the authorities having jurisdiction and in accordance with the project specification.  If noncompliance is discovered the General Contractor shall make or cause to be made all necessary corrections to meet the applicable codes and specifications.  Immediately or as directed by Lowe’s the work shall be completed without additional cost to the Lowe’s and / or the contract.

Sub-Contractor:

_______________________________________________________________

(Signature of the Authorized Agent of the Sub-Contractor)

- _______________________________________________________________

(Print Name of the Authorized Agent of the Sub-Contractor)

This Page Left Intentionally Blank