SECTION 06100 - ROUGH CARPENTRY

GENERAL

RELATED DOCUMENTS

Drawings and general provisions of the contract, including General and Supplementary Conditions and Division 1 Specification Sections, apply to this Section.

SUMMARY

This Section includes the following:

Wood blocking.

Wood nailers.

Plywood backing panels.

PRODUCTS

MANUFACTURERS

In other Part 2 articles where subparagraph titles below introduce lists, the following requirements apply for product selection:

Available Manufacturers:  Subject to compliance with requirements, manufacturers offering products that may be incorporated into the Work include, but are not limited to, the manufacturers specified.

NON TREATED MATERIALS

Application: Items indicated on Drawings, and the following:

Wood cants, nailers, curbs, equipment support bases, blocking, stripping, and similar members in connection with all roofing, flashing, vapor barriers, and waterproofing.

Material shall be Kiln-Dried (Southern Pine, Douglas Fir) structural grade #2 or better. Material shall be a min. of nominal 2” x 4’. Wood nailers shall be securely fastened to resist a minimum force of 200 lb/ft. (890 N) in any direction.

WOOD-PRESERVATIVE-TREATED MATERIALS

Preservative Treatment by Pressure Process:  AWPA C2 (lumber), except that lumber that is not in contact with the ground and is continuously protected from liquid water may be treated according to AWPA C31 with inorganic boron (SBX).

Kiln-dry material after treatment to a maximum moisture content of 19 percent.

Mark each treated item with treatment quality mark of an inspection agency approved by the American Lumber Standards Committee Board of Review.

Wood sills, sleepers, blocking, furring, stripping, and similar concealed members in contact with masonry or concrete. (non-roof related areas)

WOOD TREATMENT

All treated wood shown on the drawings shall be Grade No. 2 or better lumber, pressure treated for rot resistance with a salt-based preservative (“Wolmanized” or “Osmose” – Kiln Dried).  Creosote and asphaltic based preservatives are not acceptable.

PLYWOOD BACKING PANELS

Telephone and Electrical Equipment Backing Panels:  DOC PS 1, Exposure 1, C-D Plugged, in thickness indicated or, if not indicated, not less than 1/2 inch (12.7 mm) thick.  Both sides and all edges to receive intumescent coating (see Section 07812).

MISCELLANEOUS MATERIALS

Fasteners:

Where rough carpentry is exposed to weather, in ground contact, or in area of high relative humidity, provide fasteners with hot-dip zinc coating complying with ASTM A 153/A 153M.

Power-Driven Fasteners:  CABO NER-272.

Bolts:  Steel bolts complying with ASTM A 307, Grade A (ASTM F 568M, Property Class 4.6); with ASTM A 563 (ASTM A 563M) hex nuts and, where indicated, flat washers.

Metal Framing Anchors: Made from hot-dip, zinc-coated steel sheet complying with ASTM A 653/A 653M, G60 (Z180) coating designation.

Manufacturers:

Alpine Engineered Products, Inc.

Cleveland Steel Specialty Co.

Harlen Metal Products, Inc.

KC Metals Products, Inc.

Silver Metal Products, Inc.

Simpson Strong-Tie Company, Inc.

Southeastern Metals Manufacturing Co., Inc.

United Steel Products Company, Inc.

Research/Evaluation Reports:  Provide products acceptable to authorities having jurisdiction and for which model code research/evaluation reports exist that show compliance of metal framing anchors, for application indicated, with building code in effect for Project.

Allowable Design Loads:  Meet or exceed those indicated per manufacturer's published values determined from empirical data or by rational engineering analysis and demonstrated by comprehensive testing performed by a qualified independent testing agency.

EXECUTION

INSTALLATION

All scaffolding, concrete forms, rough bucks and any other items of carpentry work necessary for the completion of this building shall be done by the Contractor.  Carpentry work shall be accurately laid out and the work performed in such a manner that the adjacent trades can be easily and neatly fitted to the carpentry work.   Concrete forms must be tight and so constructed as to present neat, smooth concrete surfaces.

Set rough carpentry to required levels and lines, with members plumb, true to line, cut, and fitted.  Fit rough carpentry to other construction; scribe and cope as needed for accurate fit.  Locate furring, nailers, blocking, and similar supports to comply with requirements for attaching other construction.

Apply field treatment complying with AWPA M4 to cut surfaces of preservative-treated lumber and plywood.

Securely attach rough carpentry work to substrate by anchoring and fastening as indicated, complying with the following:

CABO NER-272 for power-driven fasteners.

Published requirements of metal framing anchor manufacturer.

Fastening Methods:  Use fasteners of appropriate type and length.  Predrill members when necessary to avoid splitting wood.

Plywood Backing Panels: Nail or screw to supports.

END OF SECTION 06100