SECTION 07840 – FIRESTOPPING

- GENERAL

  - RELATED DOCUMENTS

    - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 1 Specification Sections, apply to this Section.

  - SUMMARY

    - This Section includes sealants for the following:

      - Firestop system through fire resistance - rated walls.

    - See Division 15 Mechanical Penetrations through Rated Assemblies.

    - See Division 16 Electrical Penetrations through Rated Assemblies.

  - SUBMITTALS

    - Firestop System Submittals:  For each through-penetration firestop system, show each kind of construction condition penetrated, relationships to adjoining construction and kind of penetrating item.  Include firestop design designation of testing and inspecting agency acceptable to authorities having jurisdiction that evidence compliance with requirements for each condition.

  - QUALITY ASSURANCE

    - Applicator shall be licensed by the manufacturer of  fireproofing materials.

    - Conform to flame and temperature ratings established by and ASTM E814.

    - Conform to local Building Code.

    - Submit manufacturer’s certification that materials meet or exceed specified requirements.

    - Maintain flame and temperature ratings equal to surrounding material.

  - WARRANTY

    - Special Installer's Warranty: Written warranty in which Installer agrees to repair or replace firestoppings that do not meet requirements specified in this Section or fail in adhesion within specified warranty period two years from date of Final Acceptance.

- PRODUCTS

  - MANUFACTURERS

    - In other Part 2 articles where subparagraph titles below introduce lists, the following requirements apply for product selection:

      - Products:  Subject to compliance with requirements, provide one of the products specified.

  - MATERIALS, GENERAL

    - Compatibility:  Provide firestopping, backings, smokeseals and other related materials that are compatible with one another and with joint substrates under conditions of service and application, as demonstrated by firestopping manufacturer based on testing and field experience.

    - Colors of Exposed Firestopping materials shall match adjacent finish unless otherwise indicated on drawings or reviewed by owner.

  - FIRESTOPPING, GENERAL

    - Compatibility:  Provide through-penetration firestop and smokeseal systems that are compatible with one another with the substrates forming openings, and with the items, if any, penetrating through-penetration firestop system under conditions of service and application, as demonstrated by through-penetration firestop system manufacturers based on testing and field experience.

    - Accessories:  Provide components for each through-penetration firestop system that are needed to install fill materials and comply with "Performance Requirements" article.  Use only components specified through penetration firestop system manufacturer and approved by the qualified testing and inspecting agency for firestop systems indicated.  Accessories include, but are not limited to, the following items:

      - Permanent forming/damming/backing materials, include the following:

        - Slat-/rock-wood-fiber insulation.

        - Sealants in combination with other forming/damming/backing materials to prevent leakage of full materials in liquid state.

        - Fire-rated form board.

        - Fillers for sealants.

      - Temporary forming materials.

      - Substrate primers.

      - Collars.

      - Steel sleeves.

      - Products:

        - 3M Fire Prevention Products.

        - Hilti Firestop Systems

        - International Protective Coatings Corporation (IPC, a Division of W.R. Grace).

        - Tremco Inc.

- EXECUTION

  - INSTALLATION

    - Contractor performing work must be one of the Firestopping Manufacturer’s Approved Applicators.

    - Surface Cleaning of Joints: Clean out joints immediately before installing firestopping.

      - Remove foreign material from joint substrates that could interfere with adhesion of joint sealant.

      - Clean porous joint substrate surfaces by brushing, grinding, blast cleaning, mechanical abrading, or a combination of these methods to produce a clean, sound substrate capable of developing optimum bond with firestoppings.  Remove loose particles remaining from above cleaning operations by vacuuming or blowing out joints with oil-free compressed air.

      - Remove laitance and form-release agents from concrete.

      - Clean nonporous surfaces with chemical cleaners or other means that do not stain, harm substrates, or leave residues could interfere with adhesion of firestoppings.

      - All surfaces to be caulked shall be clean and dry.

    - Masking Tape:  Use masking tape where required to prevent contact of sealant with adjoining surfaces that otherwise would be permanently stained or damaged by such contact or by cleaning methods required to remove sealant smears.  Remove tape immediately after tooling without disturbing joint seal.

    - Firestopping Installation:  Comply with manufacturer’s recommendations for use of firestopping as applicable to materials, applications, and conditions indicated.

    - Clean excess or incidental firestopping material as installation progresses by methods and with cleaning materials approved in writing by manufacturers of firestopping material and of other products involved in cleaning process.

    - Through-Penetration Firestop Installation:

      - General:   Installation through-penetration systems to comply with "Performance Requirements" of the firestop system manufacturer's written installation instructions and published drawings for the applications indicated.

END OF SECTION 07840

# CONFORMANCE SUBMITTAL

# Section 07840 –Firestopping and Smokeseals

Lowe’s of 		________________________________________________________________

(City, Province)

General Contractor:	________________________________________________________________

(Company Name, Phone Number)

________________________________________________________________

(Address)

Sub-Contractor:		________________________________________________________________

(Company Name, Phone Number)

________________________________________________________________

(Address)

The following product has been selected (check one box) for use in this project from the list of acceptable products specified:

Firestopping:

3M Fire Preventive Products.

Hilti Firestop Systems.

International Protective Coatings Corporation

Tremco Inc.

I represent to Lowe’s that the product selected will be installed in compliance with the applicable codes for the authorities having jurisdiction and in accordance with the project specification.  If noncompliance is discovered the General Contractor shall make or cause to be made all necessary corrections to meet the applicable codes and specifications.  Immediately or as directed by Lowe’s the work shall be completed without additional cost to the Lowe’s and / or the contract.

General Contractor:	________________________________________________________________

(Signature of the Authorized Agent of the General Contractor)

________________________________________________________________

(Print Name of the Authorized Agent of the General Contractor)

I represent to Lowe’s that the product selected will be installed in compliance with the applicable codes for the authorities having jurisdiction and in accordance with the project specification.  If noncompliance is discovered the General Contractor shall make or cause to be made all necessary corrections to meet the applicable codes and specifications.  Immediately or as directed by Lowe’s the work shall be completed without additional cost to the Lowe’s and / or the contract.

Sub-Contractor:		________________________________________________________________

(Signature of the Authorized Agent of the Sub-Contractor)

________________________________________________________________

- (Print Name of the Authorized Agent of the Sub-Contractor)

This Page Left Intentionally Blank