SECTION 08311 - STRIP DOORS

- GENERAL

  - RELATED DOCUMENTS

    - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 1 Specification Sections, apply to this Section.

  - SUMMARY

    - This Section includes the following:

      - Strip doors.

  - SUBMITTALS

    - Product Data:  For each type of strip doors.

    - Schedule:  Door and frame schedule, including types, general locations, sizes, construction details, and other data pertinent to installation.  Use same reference designations indicated on Drawings and Door Schedule.

  - QUALITY ASSURANCE

    - See drawings for specific opening(s) dimensions for strip application(s).

- PRODUCTS

  - MATERIALS

    - Steel Plates, Shapes, and Bars:  ASTM A 36/A 36M.

      - Hot-Dip Galvanized Steel.

    - Aluminum Sheet:  ASTM B 209 (ASTM B 209M), alloy and temper recommended by aluminum producer and finisher for type of use and finish indicated, and with not less than the strength and durability properties of alloy 5005-H15.

    - Aluminum Extrusions:  ASTM B 221 (ASTM B 221M), alloy 6063-T6.

  - STRIP DOORS

    - Manufacturers:   Subject to compliance with requirements, provide products by one of the following:

      - Strip Doors:

        - Aleco (E.S. Robbins)

        - FlexBarrier Products, Inc.

        - Frommelt Industries, Inc.

        - W.B. McGuire Company.

        - Chase Industries, Inc.

    - Furnish and install strip doors at locations shown on the Drawings.

    - Strips shall be “Low Temp” (20 deg F) non-toxic extruded PVC (12 inches wide, full lap).

    - Mounting method shall be determined by application.

    - Color:  Clear.

- EXECUTION

  - INSTALLATION

    - Advise installers of other work about specific requirements relating to strip door installation, including sizes of openings to receive door, as well as locations of supports, inserts, and anchoring devices.

    - Adjust doors and hardware after installation for proper operation.

END OF SECTION 08311

This Page Left Intentionally Blank