SECTION 09512 - TILE CEILINGS

GENERAL

RELATED DOCUMENTS

Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 1 Specification Sections, apply to this Section.

SUMMARY

This Section includes acoustical tiles, heavy duty tiles and concealed suspension systems.

SUBMITTALS

The General Contractor and Sub-Contractor shall execute the provided Conformance Submittal for the product(s) specified.

The General Contractor and the Sub-Contractor shall utilize ONLY the products as noted in this specification.  NO SUBSTITUTIONS ALLOWED.

QUALITY ASSURANCE

Fire-Test-Response Characteristics:

Acoustical Tiles:  ASTM E 1264, Class A materials, when tested per ASTM E 84.

Fire-Rating Ceiling Assemblies:  Where indicated, provide products identical to those tested for fire resistance per ASTM E 119 and listed in UL's "Fire Resistance Directory," in ITS's "Certification Listings," or in the listing of another qualified testing and inspecting agency.

Identify products with appropriate markings of applicable testing and inspecting agency.

EXTRA MATERIALS

Furnish extra materials described below that match products installed and that are packaged with protective covering for storage and identified with labels describing contents.

Ceiling Tiles:  Full-size units equal to 2 percent of amount installed for each type, color, and pattern indicated.

PRODUCTS

MANUFACTURERS

Manufacturers:  Subject to compliance with requirements, provide products by the manufacturers specified.

ACOUSTICAL TILE

Products:

Armstrong “Cortega, Square Lay-In medium texture”, white.

Celotex “Baroque, medium texture”

U.S.G. “Radar”

Acoustical tiles shall be 24” x 48” x 5/8” or 24” x 24” x 5/8” matte finish mineral fiber ceiling boards and shall conform to Federal Specification SS-S-118a, Class 25.   Light reflectance shall be no less than 75% and have NRC minimum range of .50 - .60.

HEAVY DUTY TILE

Products:

USG Corporation, SHEETROCK Brand Gypsum Lay-in Panels, vinyl faced gypsum panel, 2’ x 2’ x 1/2” thick.

SUSPENSION SYSTEM

Products:

Donn Products, Inc., Westlake, OH.

Acoustical Tile Supplier Standard Grid System.

The ceiling suspension system shall be an exposed grid system with exposed flanges having a factory applied white enamel finish with roll formed capped edges.

Main Runner:   DX-24

Cross Tees:   DX-424

Wall Angle:   M6S

Ceiling Suspension System: Direct hung; ASTM C 635, intermediate-duty structural classification.

EXECUTION

INSTALLATION

General:  Comply with Ceiling and Interior Systems Contractors Association (CISCA) "Ceiling Systems Handbook."

Measure each ceiling area and establish layout of tile units to balance border widths at opposite edges of each ceiling.  Avoid use of less-than-half-width units at borders.

Ceiling Suspension System Installation:  ASTM C 636.

Ceiling Suspension System Installation Requiring Seismic Restraint

UBC Standard 25-2.

CISCA's "Recommendations for Direct-Hung Acoustical Tile and Lay-in Panel Ceilings--Seismic Zones 0-2."

CISCA's "Guidelines for Seismic Restraint of Direct-Hung Suspended Ceiling Assemblies--Seismic Zones 3 & 4."

ASTM E 580.

Hangers:

Attach hangers to structural members.  Do not support ceilings from or attach hangers to permanent metal forms, steel deck tabs, steel roof decks, ducts, pipes, or conduit.

Install hangers plumb and free from contact with insulation or other objects within ceiling plenum that are not part of supporting structure or of ceiling suspension system.

Splay hangers only where required to miss obstructions; offset resulting horizontal forces by bracing, countersplaying, or other equally effective means.

Where width of ducts and other construction within ceiling plenum produces hanger spacings that interfere with location of hangers at spacings required to support standard suspension system members, install supplemental suspension members and hangers in form of trapezes or equivalent devices.  Size supplemental suspension members and hangers to support ceiling loads within performance limits established by referenced standards and publications.

Secure wire hangers to ceiling suspension members and to supports above with a minimum of three tight turns.  Connect hangers directly either to structures or to inserts, eye screws, or other devices that are secure; that are appropriate for substrate; and that will not deteriorate or otherwise fail due to age, corrosion, or elevated temperatures.

Space hangers not more than 48 inches (1200 mm) o.c. along each member supported directly from hangers, unless otherwise indicated; and provide hangers not more than 8 inches (200 mm) from ends of each member.

Provide additional hangers to grid at corners of light fixtures and vent locations.

Secure bracing wires to ceiling suspension members and to supports with a minimum of four tight turns.  Suspend bracing from building's structural members as required for hangers, without attaching to permanent metal forms, steel deck, or steel deck tabs.  Fasten bracing wires into concrete with cast-in-place or postinstalled anchors.

Install edge moldings and trim at perimeter of ceiling area and where necessary to conceal edges of ceiling tiles.

Apply acoustical sealant in a continuous ribbon concealed on back of vertical legs of moldings before they are installed.

Screw attach moldings to substrate at intervals not more than 16 inches (400 mm) o.c. and not more than 3 inches (75 mm) from ends, leveling with ceiling suspension system to a tolerance of 1/8 inch in 12 feet (3 mm in 3.6 m).  Miter corners accurately and connect securely.

Do not use exposed fasteners, including pop rivets, on moldings and trim.

Install suspension system runners so they are square and securely interlocked with one another.  Remove and replace dented, bent, or kinked members.

WARRANTY

The ceiling tile contractor shall guarantee all ceiling tile work included in this project for a period of one year from date of Final Acceptance.  This guarantee shall cover any sagging in the suspension system, any cracked or broken tiles or any tile failure because of poor workmanship or improperly installed materials.

CLEANING

Upon completion of this project all debris must be removed from the site and rooms left broom clean.  All tile work shall be kept clean as work progress and no broken, damaged or soiled tiles will be accepted.

END OF SECTION 09512

This Page Left Intentionally Blank

# CONFORMANCE SUBMITTAL

# Section 09512 –Tile Ceiling

Lowe’s of ____________________________________________________________________________

(City, State)

General Contractor:	________________________________________________________________

(Company Name)

________________________________________________________________

(Address, Phone Number)

Sub-Contractor:		________________________________________________________________

(Company Name)

________________________________________________________________

(Address, Phone Number)

The following product has been selected (check one box) for use in this project from the list of acceptable products specified:

Acoustical Tile:

Armstrong “Cortega, Square Lay-In medium texture”.

Celotex “Baroque, medium texture”.

USG “Radar”.

I represent to Lowe’s that the product selected will be installed in compliance with the applicable codes for the authorities having jurisdiction and in accordance with the project specification.  If noncompliance is discovered the General Contractor shall make or cause to be made all necessary corrections to meet the applicable codes and specifications.  Immediately or as directed by Lowe’s the work shall be completed without additional cost to the Lowe’s and / or the contract.

General Contractor:	________________________________________________________________

(Signature of the Authorized Agent of the General Contractor)

________________________________________________________________

(Print Name of the Authorized Agent of the General Contractor)

I represent to Lowe’s that the product selected will be installed in compliance with the applicable codes for the authorities having jurisdiction and in accordance with the project specification.  If noncompliance is discovered the General Contractor shall make or cause to be made all necessary corrections to meet the applicable codes and specifications.  Immediately or as directed by Lowe’s the work shall be completed without additional cost to the Lowe’s and / or the contract.

Sub-Contractor:		________________________________________________________________

(Signature of the Authorized Agent of the Sub-Contractor)

________________________________________________________________

(Print Name of the Authorized Agent of the Sub-Contractor)

# CONFORMANCE SUBMITTAL

# Section 09512 –Tile Ceilings

Lowe’s of ____________________________________________________________________________

(City, State)

General Contractor:	________________________________________________________________

(Company Name)

________________________________________________________________

(Address, Phone Number)

Sub-Contractor:		________________________________________________________________

(Company Name)

________________________________________________________________

(Address, Phone Number)

The following product has been selected (check one box) for use in this project from the list of acceptable products specified:

Heavy Duty Tile:

USG Corporation

I represent to Lowe’s that the product selected will be installed in compliance with the applicable codes for the authorities having jurisdiction and in accordance with the project specification.  If noncompliance is discovered the General Contractor shall make or cause to be made all necessary corrections to meet the applicable codes and specifications.  Immediately or as directed by Lowe’s the work shall be completed without additional cost to the Lowe’s and / or the contract.

General Contractor:	________________________________________________________________

(Signature of the Authorized Agent of the General Contractor)

________________________________________________________________

(Print Name of the Authorized Agent of the General Contractor)

I represent to Lowe’s that the product selected will be installed in compliance with the applicable codes for the authorities having jurisdiction and in accordance with the project specification.  If noncompliance is discovered the General Contractor shall make or cause to be made all necessary corrections to meet the applicable codes and specifications.  Immediately or as directed by Lowe’s the work shall be completed without additional cost to the Lowe’s and / or the contract.

Sub-Contractor:		________________________________________________________________

(Signature of the Authorized Agent of the Sub-Contractor)

________________________________________________________________

(Print Name of the Authorized Agent of the Sub-Contractor)