SECTION 09651 - RESILIENT TILE FLOORING

GENERAL

RELATED DOCUMENTS

Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 1 Specification Sections, apply to this Section.

SUMMARY

This Section includes the following:

Vinyl base.

Vinyl reducer strips.

SUBMITTALS

The General Contractor and the Sub-Contractor shall execute the provided Conformance Submittal for the product(s) specified.

The General Contractor and the Sub-Contractor shall utilize ONLY the products as noted in this specification.  NO SUBSTITUTIONS ALLOWED.

PROJECT CONDITIONS

Maintain a temperature of not less than 72 deg F or more than 95 deg F in spaces to receive floor tile for at least 48 hours before installation, during installation, and for at least 48 hours after installation, unless manufacturer's written recommendations specify longer time periods.

After post installation period, maintain a temperature of not less than 72 deg F or more than 95 deg F.

Install floor tile after other finishing operations, including painting, have been completed.

All concrete surfaces that are to receive vinyl composition tile shall be inspected by the installing contractor, if found unsatisfactory the work of laying the vinyl composition tile shall not commence until the defective surfaces have been made satisfactory.  Beginning work constitutes an acceptance of the base concrete flooring.

QUALITY ASSURANCE

The work covered by this division of the specifications includes the furnishing of all labor, equipment, appliances, materials and the performing of all operations in connection with the laying, installing and finishing of all vinyl composition in strict accordance with the applicable drawings, schedules, and the approved samples and color selection.

EXTRA MATERIALS

Furnish extra materials that match products installed and that are packaged with protective covering for storage and identified with labels describing contents.  Furnish one box for each 50 boxes or fraction thereof of each type, color, and pattern of floor tile.

PRODUCTS

VINYL COVE BASE

Vinyl cove base as manufactured by Armstrong, Johnsonite, Flexco or Roppe, shall be furnished and installed around all walls as indicated in the Finish Schedule.   All corners shall receive pre-molded inside/outside corner pieces matching adjacent base.  Thickness shall be .080 inch.  Base shall be adhered to the wall with Cove Base Cement per manufacturer’s specifications.

Height of the base shall be:

4 inches in all areas U.O.N.

6 inch base in Unisex / Family Room only

Vinyl Base Color:  Armstrong 64 Fresh Taupe (Lowes special order 89092), Johnsonite #80 Fawn, Flexco 037 Dark Beige or Roppe No. P140 Fawn.

VINYL REDUCER STRIP

Vinyl reducer strip to exposed concrete shall be Armstrong Shadow Gray VT5, Roppe Dark Gray No. 150 #22, or similar color by other manufacturers listed above.

INSTALLATION ACCESSORIES

Trowelable Leveling and Patching Compounds:  Latex-modified, portland cement-based or blended hydraulic cement-based formulation provided or approved by flooring manufacturer for applications indicated.

Adhesives:  Water-resistant type recommended by manufacturer to suit resilient products and substrate conditions indicated.

EXECUTION

INSTALLATION

Concrete Subfloors:  Verify that concrete slabs comply with ASTM F 710 and the following:

Slab substrates are dry and free of curing compounds, sealers, hardeners, and other materials that may interfere with adhesive bond.  Determine adhesion and dryness characteristics by performing bond and moisture tests recommended by flooring manufacturer.

Before installing floor tile:

Remove coatings, including curing compounds, and other substances that are incompatible with flooring adhesives and that contain soap, wax, oil, or silicone, using mechanical methods.  Do not use solvents.

Floors must be dry, smooth, free of oil, grease, lumps or foreign matter.  Specific instructions as published by the tile manufacturer must be followed.  The floor tile shall be installed in a grid non-directional pattern.

Use trowelable leveling and patching compounds to fill cracks, holes, and depressions in substrates.

Move resilient floor tile and installation accessories into spaces where they will be installed at least 48 hours before installation, unless longer conditioning periods are recommended in writing by manufacturer.  Install products only after they are at the same temperature as the space where they are to be installed.

Broom and vacuum clean substrates to be covered immediately before product installation.  After cleaning, examine substrates for moisture, alkaline salts, carbonation, or dust.  Proceed with installation only after unsatisfactory conditions have been corrected.

Lay out tiles from center marks established with principal walls, discounting minor offsets, so tile widths at opposite edges of room are equal to one another and are at least one-half of a tile.

Lay tiles square with room axis, unless otherwise indicated.

Match tiles for color and pattern by selecting tiles from cartons in same sequence as manufactured and packaged.

Pattern:   Non-directional (grid pattern quarter turn).

Scribe, cut, and fit tiles to butt neatly and tightly to vertical surfaces and permanent fixtures, including built-in furniture, cabinets, pipes, outlets, edgings, door frames, thresholds, and nosings.

Extend tiles into toe spaces, door reveals, closets, and similar openings.

Maintain reference markers, holes, and openings that are in place or marked for future cutting by repeating on finish flooring as marked on subfloor.  Use chalk or other nonpermanent, non-staining marking device.

Install tiles on covers for telephone and electrical ducts, and similar items in finished floor areas.  Maintain overall continuity of color and pattern with pieces of flooring installed on covers.  Tightly adhere edges to perimeter of floor around covers and to covers.

Adhere tiles to flooring substrates using a full spread of adhesive applied to substrate to produce a completed installation without open cracks, voids, raising and puckering at joints, telegraphing of adhesive spreader marks, and other surface imperfections.

CLEANING

Immediately after installing resilient floor tile, the subcontractor for vinyl composition shall collect and remove from the site all debris resulting from vinyl composition installation and clean the floor to make it ready for sealing.

The use of acids in any form is prohibited.  The installing contractor will remove will excess adhesives and debris from floors using proper cleaning solutions as recommended by the manufacturer.

END OF SECTION 09651

# CONFORMANCE SUBMITTAL

# Section 09651 – Resilient Tile Flooring

Lowe’s of ____________________________________________________________________________

(City, State)

General Contractor:	________________________________________________________________

(Company Name)

________________________________________________________________

(Address, Phone Number)

Sub-Contractor:		________________________________________________________________

(Company Name)

________________________________________________________________

(Address, Phone Number)

The following product has been selected (check one box) for use in this project from the list of acceptable products specified:

Vinyl Cove Base:

Armstrong #64 Fresh Taupe (Lowes Special Order 89092)

Johnsonite #80 Fawn

Flexco #037 Dark Beige

Roppe #P140 Fawn.

I represent to Lowe’s that the product selected will be installed in compliance with the applicable codes for the authorities having jurisdiction and in accordance with the project specification.  If noncompliance is discovered the General Contractor shall make or cause to be made all necessary corrections to meet the applicable codes and specifications.  Immediately or as directed by Lowe’s the work shall be completed without additional cost to the Lowe’s and / or the contract.

General Contractor:	________________________________________________________________

(Signature of the Authorized Agent of the General Contractor)

________________________________________________________________

(Print Name of the Authorized Agent of the General Contractor)

I represent to Lowe’s that the product selected will be installed in compliance with the applicable codes for the authorities having jurisdiction and in accordance with the project specification.  If noncompliance is discovered the General Contractor shall make or cause to be made all necessary corrections to meet the applicable codes and specifications.  Immediately or as directed by Lowe’s the work shall be completed without additional cost to the Lowe’s and / or the contract.

Sub-Contractor:		________________________________________________________________

(Signature of the Authorized Agent of the Sub-Contractor)

________________________________________________________________

(Print Name of the Authorized Agent of the Sub-Contractor)