SECTION 10155 - TOILET COMPARTMENTS

- GENERAL

  - RELATED DOCUMENTS

    - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 1 Specification Sections, apply to this Section.

  - SUMMARY

    - Furnish and install Phenolic toilet compartments and urinal screen as shown on the drawings and as specified herein.

  - SUBMITTALS

    - Product Data: For each product indicated, including a color chart.

    - Shop Drawings: Include dimensioned plans and elevations, sections, details of installation, and attachments to other work to Site / Folio (www.sitefolio.net).

- PRODUCTS

  - MANUFACTURER

    - Global:

For Global products contact Cook and Boardman - Joe Harrell, Director National Business Development (336-413-7050) or Josh Wilkinson (336-837-0673 ext. 3819)

  - MATERIALS  -  SOLID PHENOLIC CORE PARTITIONS

    - Partitions to be Global solid phenolic black core, floor mounted  and overhead braced.

    - Urinal screens to be supported by floor braced pilaster.

    - Toilet compartments and urinal screens shall be composed of compressed cellulose fibers resin-impregnated core.   Plastic Laminate skin shall be Formica, Navy Grafix #7018-58.

    - Doors and partitions are manufactured from solid phenolic materials. Laminating to a core material is not an acceptable alternative. All edges are beveled and polished.  Phenolic is Class-B fire rated based on E-84 Tunnel testing; NFPA No. 255 and U.L. No. 723.

    - Pilasters are manufactured from solid phenolic materials. Laminating to a core material is not an acceptable alternative. All edges are beveled and polished.  Phenolic is Class-B fire rated based on E-84 Tunnel testing; NFPA No. 255 and U.L. No. 723. Pilasters will be anchored to concrete floor Using 3/8” threaded rods, hex nuts and washers to provide floor adjustment.  All pilasters will have a  4”  (101.6) high #4 finish stainless steel plinth (type 304) and have straight, flat sides to match pilaster profile. Anodized aluminum headrail, .050" (1.27) wall thickness, with anti-grip profile, will brace the top of the pilasters.

    - Mounting brackets are heavy duty aluminum, 6063 T5 alloy, with a clear anodized, satin finish.  All connections must be continuous brackets.

    - Each compartment will be complete with all stainless steel surface mounted hardware including; slide latch, stop and keeper, coat hook, as well as all the necessary fittings and fastenings for a complete installation. Fasteners are stainless steel through bolts with tamper-proof  Torx-Pin heads. Door hinges are fully adjustable, heavy-duty cast stainless steel, wrap around vault.  Adjustable door positioning gravity cam and pintle to be made from stainless steel operating on a ball bearing assembly.  Strike & keeper to be heavy duty cast stainless steel with brushed finish. Coat hook & bumper to be 304stainless steel.

- EXECUTION

  - INSTALLATION

    - Install units rigid, straight, level, and plumb, with not more than 1/2 inch (13 mm) between pilasters and panels and not more than 1 inch (25 mm) between panels and walls.  Provide brackets, pilaster shoes, bracing, and other components required for a complete installation.  Use theft-resistant exposed fasteners finished to match hardware.  Use machine bolt and sleeve for through-bolt applications.

      - Continuous Brackets:  Align brackets at pilasters with brackets at walls.  Locate wall brackets so holes for wall anchors occur in masonry joints.

      - Set hinges on in-swinging doors to hold open approximately 30 degrees from closed position when unlatched.  Set hinges on out-swinging doors and swing doors in entrance screens to return to fully closed position.

    - Installation shall be in accordance with manufacturer’s shop drawings, as reviewed by the Owner.

    - Pilasters will be anchored to the floor using 3/8” threaded rods, hex nuts and washer to provide floor adjustment.

    - The general contractor shall inspect every door, pilaster, partition and accessory upon delivery to the job site to insure its workmanship, quality, and that the material is free from all defects.  The Owner will not accept any materials damaged and/or repaired.

END OF SECTION 10155