SECTION 10425 - SIGNS

GENERAL

RELATED DOCUMENTS

Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 1 Specification Sections, apply to this Section.

SUMMARY

This Section includes the following:

Interior signs.

SUBMITTALS

Product Data: For each product indicated, including a color chart.

Shop Drawings: Include plans, elevations, sections, details of installation, and attachments to other Work to Site|Folio (www.sitefolio.net)

PRODUCTS

MATERIALS/MANUFACTURERS

All signs:

All text shall be accompanied by Grade 2 Braille.  Braille shall be separated ½” from the corresponding raised characters or symbols.  Grade 2 Braille translation to be provided by signage manufacturer.

All letters, numbers and/or symbols shall contrast with their background, either light characters on a dark background or dark characters on a light background.  Characters and background shall have a non-glare finish.

Plaque material shall be NEMA rated and have flammability and smoke values that meet the standards for flammability of interior materials.

Acceptable Manufacturers:

Allen Industries, Inc., (888) 294-2007

ASI Sign Systems.(919) 544-2223

Mohawk Sign Systems. (518) 370-3433

All signage shall meet all Federal, State and Local Codes.

Interior Sign Package:

| - Quantity: - 2 - 2 - 1 - 1 - 1 | - Size: - 8” x 12” - 8” x 12” - 3” x 12” - 3” x 12” - 8” x 12” | - Description: - Men w/handicapped symbol - Women w/handicapped symbol - Store Manager - Training Room - Unisex Family Toilet w/ handicapped symbol | - Color: - Navy Blue - Navy Blue - Navy Blue - Navy Blue - Navy Blue |
| --- | --- | --- | --- |

- Additional Interior Signage (If indicated on drawings):

| - Quantity: - 1 per each door | - Size: - 1” vinyl lettering | - Description: - “This door to remain open when the building is occupied” | - Color: - Red |
| --- | --- | --- | --- |

EXECUTION

INSTALLATION

Install signs level, plumb, and at height indicated, with sign surfaces free from distortion or other defects in appearance.

Wall-Mounted Panel Signs:

Vinyl-Tape Mounting:  Use double-sided foam tape to mount signs to smooth, nonporous surfaces.  Do not use this method for vinyl-covered or rough surfaces.

The contractor shall provide and install the sign package described herein.  The signs shall be mounted adjacent to (8 inches off) the latch side of the interior door at 5’-0” above finish floor unless noted otherwise.

Mounting requirements may vary depending on applicable codes.

After installation, clean soiled signs surface according to manufacturer’s instructions.  Protect from damage until acceptance.

END OF SECTION 10425