SECTION 10530 – ALUMINUM CANOPY

GENERAL

RELATED DOCUMENTS

Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 1 Specification Sections, apply to this Section.

SUMMARY

This Section includes the following:

Aluminum Canopy

SUBMITTALS

The General Contractor and the Sub-Contractor shall execute the provided Conformance Submittal for the product(s) specified.

If General Contractor and the Sub-Contractor submit an equal product for approval as noted in this specification full shop drawings are to be submitted.

PRODUCTS

ALUMINUM CANOPY

Aluminum canopy shall be capable of sustaining severe icing, snow drifting, hail, and hurricane winds. Aluminum canopy shall be designed to meet loading imposed by ground snow load and drift provisions as required by the prevailing building code; and any heavier or additional loadings as required by the AHJ.  If required by the AHJ, engineer sealed calculations shall be provided at no additional cost to the owner.  Deck screws shall be stainless steel, sealed with neoprene “O” ring. Deck shall be supported by structural fascia and overhead hanger system. Bracket hangers shall be spaced at 39” centers (max.) and shall be connected to masonry with ½” thru-bolts with 3” x 3” x 3/16” plate washers on inside face of masonry; unless local design loadings dictate closer spacing of hangers, larger bolts or washers. Slope hangers at 45 degrees (min.) from horizontal. All fasteners, washers, etc. shall be zinc plated per ASTM B 633.  All decking, exposed fascia members, and brackets shall have factory baked enamel finish  (color – white, unless noted otherwise on drawings),

Warranty:

Product  Install - 1 year

Finish – Standard manufacturer warranty

Canopy shall have the following minimum load capabilities:

Wind:  90 MPH

Load:  40 PSF

MANUFACTURERS

Ballew’s Aluminum Products – 2 Shelter Drive, Greer, SC 29650 – 800-231-6666; 864-272-4453 email: www.ballews.com

Metals USA – Dallas, 1100 Chase Road Ste. 100, Mesquite, TX 75149, 866-295-9016; email: www.buildingproductsusa.com (canopy manufacturer, installed by general contractor)

Mapes Architectural Canopies, 7748 N 56th Street, Lincoln NE 68514; 888-273-1132; email: www.mapescanopies.com

Metal-Era, Inc. 1600 Airport Road, Waukesha, WI 53188; 800-558-2162; email: jami.spice@metalera.com and/or jesaundra.musial@metalera.com

EXECUTION

INSTALLATION

Install aluminum canopies in strict accordance with the manufacturer’s recommendations and this specification.  The most restrictive requirements between this specification and the manufacturer’s requirements shall prevail.  Maintain a copy of installation instructions of site for reference.

END OF SECTION 10530

This Page Left Intentionally Blank

# CONFORMANCE SUBMITTAL

# Section 10530 – Aluminum Canopy

Lowe’s of ________________________________________________________________________

(City, State)

General Contractor:	________________________________________________________________

(Company Name, Phone Number)

________________________________________________________________

(Address)

Sub-Contractor:		________________________________________________________________

(Company Name, Phone Number)

________________________________________________________________

(Address)

The following product has been selected (check one box) for use in this project from the list of acceptable products specified:

Aluminum Canopy:

Ballew’s Aluminum Products

Metals USA

Mapes Architectural Canopies

Metal-Era, Inc

I represent to Lowe’s that the product selected will be installed in compliance with the applicable codes for the authorities having jurisdiction and in accordance with the project specification.  If noncompliance is discovered the General Contractor shall make or cause to be made all necessary corrections to meet the applicable codes and specifications.  Immediately or as directed by Lowe’s the work shall be completed without additional cost to the Lowe’s and / or the contract.

General Contractor:	________________________________________________________________

(Signature of the Authorized Agent of the General Contractor)

________________________________________________________________

(Print Name of the Authorized Agent of the General Contractor)

I represent to Lowe’s that the product selected will be installed in compliance with the applicable codes for the authorities having jurisdiction and in accordance with the project specification.  If noncompliance is discovered the General Contractor shall make or cause to be made all necessary corrections to meet the applicable codes and specifications.  Immediately or as directed by Lowe’s the work shall be completed without additional cost to the Lowe’s and / or the contract.

Sub-Contractor:		________________________________________________________________

(Signature of the Authorized Agent of the Sub-Contractor)

________________________________________________________________

- (Print Name of the Authorized Agent of the Sub-Contractor)