MASTERSPEC Short Form Copyright 1999, The American Institute of Architects (AIA)

SECTION 16140 - WIRING DEVICES

GENERAL

SUMMARY

This Section includes:

Adjust list below to suit Project.

Receptacles.

Connectors.

Switches.

Finish plates.

SUBMITTALS

Completed “Conformance Submittal” included with this Section.

Retain first paragraph below if products have critical features needing hands-on appraisal.

QUALITY ASSURANCE

Electrical Components, Devices, and Accessories:  Listed and labeled as defined in NFPA 70, Article 100, by a testing agency acceptable to authorities having jurisdiction, and marked for intended use.

Comply with NEMA WD 1.

Comply with NFPA 70.

PRODUCTS

MANUFACTURERS

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.

Available Manufacturers:  Subject to compliance with requirements, manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following:

Retain above for nonproprietary or below for semiproprietary specification.  Refer to Division 1 Section "Product Requirements."

Wiring Devices:

Bryant Electric, Inc.

Eagle Electric Manufacturing Co., Inc.

GE Company; GE Wiring Devices.

Hubbell, Inc.; Wiring Devices Div.

Killark Electric Manufacturing Co.

Leviton Manufacturing Co., Inc.

Pass & Seymour/Legrand; Wiring Devices Div.

Multioutlet Assemblies:

Airey-Thompson Co.

Wiremold.

RECEPTACLES

See Editing Instruction No. 3 in the Evaluations for wiring device grades.

Straight-Blade and Locking Receptacles: General-Duty grade, NEMA 5-20R.

GFCI Receptacles:  Feed-through type, with integral NEMA WD 6, Configuration 5-20R duplex receptacle arranged to protect connected downstream receptacles on same circuit.  Design units for installation in a 2-3/4-inch- deep outlet box without an adapter.

SWITCHES

Snap Switches: Specification Grade, quiet type.

FINISH COLOR FOR RECEPTACLES, MOTION SENSORS AND SWITCHES

See Editing Instruction No. 2 in the Evaluations for a discussion of wiring device colors.Coordinate with Drawings.

Color: Gray for gray walls and ivory for walls other than gray, unless otherwise indicated or required by Code.

Isolated Ground Receptacle Color:  Orange with contrasting triangle.

WALL PLATES

Single and combination types match corresponding wiring devices.

Plate-Securing Screws: Metal with head color to match plate finish.

Coordinate first subparagraph below with Division 9 Section "Painting."

Material for Finished Spaces: 0.04-inch- thick, Type 302, satin-finished stainless steel.

Material for Unfinished Spaces: 0.04-inch- thick, Type 302, satin-finished stainless steel.

EXECUTION

INSTALLATION

Install devices and assemblies plumb and secure.

Install wall plates when painting is complete.

Coordinate first two paragraphs below with Drawings.

Arrangement of Devices: Unless otherwise indicated, mount flush, with long dimension vertical, and grounding terminal of receptacles on top.  Group adjacent switches under single, multigang wall plates.

Protect devices and assemblies during painting.

FIELD QUALITY CONTROL

Test wiring devices for proper polarity and ground continuity.  Operate each device at least six times.

Test GFCI operation with both local and remote fault simulations according to manufacturer's written instructions.

Replace damaged or defective components.

END OF SECTION 16140

# CONFORMANCE SUBMITTAL

# Section 16140 – Wiring Devices

Lowe’s of _______________________________________________________________________

(City, State)

General Contractor:	________________________________________________________________

(Company Name, Phone Number)

________________________________________________________________

(Address)

Sub-Contractor:		________________________________________________________________

(Company Name, Phone Number)

________________________________________________________________

(Address)

# Product Verification:

The wiring devices installed in this store will meet the product guidelines of Section 16140 “Wiring Devices”

General Contractor’s Affidavit:

I represent to Lowe’s that the product selected will be installed in compliance with the applicable codes for the authorities having jurisdiction and in accordance with the project specification.  If noncompliance is discovered the General Contractor shall make or cause to be made all necessary corrections to meet the applicable codes and Lowe’s specifications.  The work shall be completed immediately as directed by Lowe’s without additional cost to Lowe’s and/or the contract.

General Contractor:	________________________________________________________________

(Signature of the Authorized Agent of the General Contractor)			(Date)

________________________________________________________________

(Print Name of the Authorized Agent of the General Contractor)

Sub-Contractor’s Affidavit:

I represent to Lowe’s that the product selected will be installed in compliance with the applicable codes for the authorities having jurisdiction and in accordance with the project specification.  If noncompliance is discovered the Sub-Contractor shall make or cause to be made all necessary corrections to meet the applicable codes and Lowe’s specifications.  The work shall be completed immediately as directed by Lowe’s and/or the General Contractor without additional cost to Lowe’s and/or the contract.

Sub-Contractor:		________________________________________________________________

(Signature of the Authorized Agent of the Sub-Contractor)				(Date)

________________________________________________________________							              (Print Name of the Authorized Agent of the Sub-Contractor)

This Page Left Intentionally Blank